# Design Project:

# Test Data

Name: Enze Xu	Student ID:  A01336393

### Linear Power Supply – Parameter Test [2]

Recall the Non-Ideal Voltage Supply and DC Voltage Minimum Equation

VDC=Vm-ILRO,  VDC,min=Vm-ILRO-ΔVp-p2,  ΔVp-p=IL2fC

This test finds the values for RO and ΔVp-pIL to model this power supply.

- Power the power supply using 12.5 V from the 120/25V CT transformer.

- Place an Ammeter between the Unregulated Voltage Out of the Power Supply and the load.

- Use an oscilloscope across the load to measure the average and cursors to measure the peaks.

Table 1: Linear Power Supply - Parameter Test Data

| Load Ω | Measured Iout mA | Measured VDC | Calculated RO | Measured VP(max) | Measured VP(min) | Calculated ΔVp-p/IL |
| --- | --- | --- | --- | --- | --- | --- |
| ∞ | 0 | 16.27 | ∞ | 16.55 | 15.89 | ∞ |
| 100 | 162 | 15.26 | 5.432 | 16.14 | 14.48 | 0.01024 |
| 50 | 310 | 14.77 | 3.742 | 15.93 | 13.46 | 0.00797 |

Average RO = 4.437

Average ΔVp-p/IL  = 0.009107

| Notes: | Collaborated with Damon Park, Justin Tan, Data collected by Enze Xu. |
| --- | --- |

### Voltage Regulator – Limits Test [2]

This test collects data on the voltage and current limit ranges.

- Power the Voltage Regulator with Vin=30 V.

- Place a voltmeter across the Voltage Regulator Output and an ammeter at the input.

- Measure and record Vmax and Vmin by adjusting the potentiometer. Ensure the voltage regulating LED is on for both limits.

- Measure the Quiescent current at each limit.

- Decrease Vin=10 V.

- Replace the voltmeter with an ammeter. This will short the output.

- Measure and record Imax and Imin by adjusting the potentiometer. Ensure the current regulating LED is on for both limits.

Table 2: Voltage Regulator -Limits Test Data

| Measured Vmin V | Measured Vmax V | Measured IQ@Vmin mA | Measured IQ@Vmax mA | Measured Imin mA | Measured Imax mA |
| --- | --- | --- | --- | --- | --- |
| 6.85 | 25.03 | 19.19 | 3.60 | 52.75 | 57.40 |

| Notes: | Collaborated with Damon Park, Justin Tan, Data collected by Enze Xu. |
| --- | --- |

### Voltage Regulator - Voltage Regulation Test [2]

This test validates the operation of voltage regulation at Vmin under varying input power.

- Power the Voltage Regulator with Vin=30 V.

- Place one voltmeter across the Voltage Regulator Output and one voltmeter across the input.

- Set Vout=15 V using the potentiometer.

- Record the voltage output for each voltage input target.

Table 3: Voltage Regulation Test Data at Vmin

| Target Vin V | Measured Vin V | Measured Vout V | Calculated Vin-Vout V |
| --- | --- | --- | --- |
| 30 | 30.38 | 14.98 | 15.4 |
| 27 | 27.00 | 14.44 | 12.89 |
| 24 | 23.96 | 13.97 | 10.02 |
| 21 | 20.96 | 13.47 | 7.47 |
| 18 | 17.97 | 12.92 | 5.06 |
| 15 | 14.90 | 12.22 | 2.68 |
| 12 | 11.92 | 11.03 | 0.87 |
| 9 | 9.04 | 8.47 | 0.54 |
| 6 | 5.97 | 5.42 | 0.51 |
| 3 | 3.08 | 2.64 | 0.44 |

| Notes: | Collaborated with Damon Park, Justin Tan, Data collected by Enze Xu. |
| --- | --- |

### Dynamic Loading Test [4]

WARNING:

Do not let the Voltage Regulator overheat.

Take measurements quickly and turn-off between measurements .

This test validates the operation of the voltage and current regulation with varying load.

- Power the power supply circuit with the 12.5 V from the 120/25V CT transformer.

- Power the Voltage Regulator with the unregulated voltage and set VLim, and ILim.

- Add a voltmeter and ammeter to the output along with one oscilloscope probe at the input and another at the output.

- Record the voltage output, current output, and the input and output ripple.

- Add two 470 Ω resistors in parallel to the output and repeat step 3.

- Repeat step 3 and 4 to complete the table.

Vin=VUnreg.,  VLim=10 V,  ILim=200 mA,  RL=470 Ω×16

Table 6: Dynamic Loading Test Data

| # of parallel 470 Ω | RL [Ω] | Measured Vout V | Measured Iout A | Measured ΔVin(p-p) V | Measured ΔVout(p-p)V |
| --- | --- | --- | --- | --- | --- |
| 0 | ∞ | 10.0 | 0 | 0.805 | 0.615 |
| 2 | 235 | 9.60 | .040 | 1.007 | 0.823 |
| 4 | 117.5 | 9.33 | .072 | 1.410 | 0.823 |
| 6 | 78.3 | 9.09 | .110 | 1.410 | 0.823 |
| 8 | 58.8 | 9.37 | .154 | 1.812 | 0.823 |
| 10 | 47 | 8.48 | .180 | 1.811 | 0.823 |
| 12 | 39.2 | 7.24 | .183 | 1.813 | 0.823 |
| 14 | 33.6 | 6.29 | .187 | 2.018 | 0.823 |
| 16 | 29.4 | 5.60 | .189 | 2.018 | 0.612 |

| Notes: | Collaborated with Damon Park, Justin Tan, Data collected by Enze Xu. |
| --- | --- |

### Motor Driver Operation Test [3]

This test validates the operation of the voltage and current regulation.

- Connect the load RL to the motor driver output.

- Arrange the meter such that they read positive when D = 0.

- Toggle the signals B, D, and S and record the power in and out.

- Record any observations in motor behavior.

Vin=12 V,  Logic 0=0 V,  Logic 1=3.3 V,  RL=Motor

Table 7: Motor Driver Operation Test Data

| BV | DV | SV | Measured Vin V | Measured Iin V | Measured Vout V | Measured Iout A | Calculated Loss PLoss [W] | Calculated Efficiency η [%] | Motor Behavior |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 12.38 | 1.62 | 0.599 | 0.49 | 19.76209 | 1.46% | No turning, no brake torque |
| 0 | 0 | 1 | 12.36 | 99.50 | 10.29 | 80.38 | 402.7098 | 67.25% | Motor turns |
| 0 | 1 | 0 | 12.37 | 1.67 | 0.592 | -0.52 | 20.35006 | 1.49% | No turning, no brake torque |
| 0 | 1 | 1 | 12.36 | 117.18 | -9.10 | -100.69 | 532.0658 | 63.26% | Motor turns in opposite direction |
| 1 | 0 | 0 | 12.39 | 1.70 | 0.589 | 0 | 21.063 | 0.00% | Motor Stop hard to turn in one direction by hand |
| 1 | 0 | 1 | 12.39 | 1.70 | 0.589 | 0 | 21.063 | 0.00% | Motor Stop hard to turn in one direction by hand |
| 1 | 1 | 0 | 12.39 | 1.77 | 0.589 | 0 | 21.9303 | 0.00% | Motor Stop hard to turn in the other direction by hand |
| 1 | 1 | 1 | 12.39 | 1.78 | 0.590 | 0 | 22.0542 | 0.00% | Motor Stop hard to turn in the other direction by hand |

| Notes: | For cases when the motor isn’t moving. Spin the motor by hand to verify if braking torque is present. Collaborated with Damon Park, Justin Tan, Data collected by Enze Xu. |
| --- | --- |

### Maximum PWM Frequency Test [2]

This test validates the maximum PWM frequency.

- Connect the load RL to the motor driver output.

- Use a function generator to generate a Pulse signal from 0 V to 3.3 V at 1 kHz.

- Using an oscilloscope, measure S and either M1 or M2 (one will toggle with S).

- Adjust the frequency to match duty cycle error.

Vin=12 V,  B=0 V,  D=0 V,  %Duty=50%,  RL=1 kΩ

Table 8: Maximum PWM Frequency Test Data

| Target %Duty Error | Measured %Dutyin % | Measured %Dutyout % | Measured Frequency Hz |
| --- | --- | --- | --- |
| 1% | 50.0 | 51.4 | 862 |
| 2% | 50.0 | 52.2 | 1718 |
| 5% | 50.0 | 54.8 | 4300 |
| 7% | 50.0 | 57.2 | 6022 |
| 10% | 50.0 | 60.0 | 8596 |

| Notes: | Collaborated with Damon Park, Justin Tan, Data collected by Enze Xu. |
| --- | --- |

### Control Signal Current Draw Test [1]

This test measures the current draw from the control signals intended to be provided by the MSP430.

- Connect the load RL to the motor driver output.

- Using an ammeter, measure the current drawn by each signal when high.

- Have all control signals and the 3v3 input provided by one source. Place an ammeter in series.

Vin=12 V,  B=3.3 V,  D=3.3 V,  S=3.3 V,  RL=Motor

IB=3.893,  ID=0.591,  IS=0.743,  I3V3=19.545 ,  ITotal=24.178

| Notes: | Collaborated with Damon Park, Justin Tan, Data collected by Enze Xu. |
| --- | --- |

### Photo of the Power Supply  [1]

### Photo of the Voltage Regulator [1]

### Photo of the Motor Driver [1]

### Photo of the Full System [1]

| Notes: |  |
| --- | --- |